IMPLEMENTATION OF THE STRATEGIC ACTION PROGRAMME (SAP) OF THE DINARIC KARST AQUIFER SYSTEM: IMPROVING GROUNDWATER GOVERNANCE AND SUSTAINABILITY OF RELATED ECOSYSTEMS

OUTPUT 1.3

BILATERAL AGREEMENT ALBANIA-MONTENEGRO:

Proposed amendments for groundwater purposes

APRIL 2026

Table of content

1. Introduction	1

2. Purpose, Scope and Methodology	2

3. Current Bilateral Framework (2018 Agreement)	2

4. Diagnostic: Groundwater and GDE Governance Gaps	5

4.1 Legal Clarity and Scope Gaps	5

4.2 Monitoring and Data Gaps	6

4.2.1 Joint Monitoring Deficit	6

4.2.2 Data Exchange and Interoperability Gaps	6

4.2.3 Early Warning and Incident Response Gaps	7

4.3 Planning Integration and Spatial Governance Gaps	7

4.4 Institutional Capacity and Compliance Gaps	8

4.5 Synthesis of Governance Gaps	8

5. Evidence from the Cemi/Cijevna TBA: why amendments are needed?	8

5.1 Karst Vulnerability and Transboundary Hydrogeological Connectivity	9

5.2 Drinking Water Dependency and Recharge Area Governance	10

5.3 Pressure Hotspots: Local Activities with Transboundary Effects	10

5.3.1 Peri-Urban and Rural Sanitation Gaps	10

5.3.2 Development Nodes	11

5.3.3 Diffuse Agricultural Pressures	11

5.3.4 Identification of Illegal or Unregistered Abstractions	11

5.4 Governance Frictions and Agreement Limitations	11

5.5 Conclusion: Evidence-Based Justification for Amendments	12

6. Objectives for an Agreement Addendum/Protocol	12

7. Proposed Legal Amendments and Key Action Points	14

8. Implementation Roadmap (Phased, 36 months)	15

9. Risks and Mitigation Measures	16

10. Conclusions and Next Steps	17

PREPARED BY DIKTAS II PROJECT TEAM

Raya Marina Stephan

Neda Dević

Halit Kamberi

# 1. Introduction

The 2018 Framework Agreement between Montenegro and the Council of Ministers of the Republic of Albania on Mutual Relations in Transboundary Water Resources Management establishes a general framework for cooperation in the management of shared water resources.

It reflects key principles of international water law, including equitable and reasonable use, prevention of transboundary harm, and institutional cooperation through a Joint Commission.

The Cemi/Cijevna basin forms part of the Dinaric karst system, characterized by rapid infiltration, limited natural filtration capacity, and strong surface-groundwater connectivity. Hydrogeological linkages between recharge areas and abstraction zones-particularly in relation to the Zeta Plain aquifer supplying Podgorica and surrounding municipalities (Zeta and Tuzi) - demonstrate that groundwater management challenges are transboundary in nature.

The BGGDA findings highlight several governance deficits: lack of a jointly validated conceptual hydrogeological model; absence of harmonized monitoring networks and QA/QC procedures; incomplete identification and protection of recharge zones; limited interoperability of groundwater data; and insufficient institutional coordination at operational level. Similar governance frictions have been observed in other Dinaric karst TBAs, reinforcing the structural nature of the problem.

In light of these findings, there is a justified need to amend or supplement the existing bilateral Agreement in order to explicitly recognize shared groundwater bodies and GDEs, establish joint monitoring and data exchange mechanisms, coordinate cross-border sanitary protection zones (SPZ) principles in karst areas, and institutionalize groundwater-specific cooperation under the Joint Commission framework. The development of this cooperation between Albania and Montenegro would take place in coordination with the Consultation and Information Exchange body, as proposed in the  Draft Multilateral Agreement.

This paper provides a legal-technical foundation for amending or supplementing the 2018 Agreement through a dedicated Addendum or Protocol focused on transboundary groundwater governance.

# 2. Purpose, Scope and Methodology

The purpose of this paper is to assess the 2018 water cooperation agreement between Albania and Montenegro regarding its implementation to transboundary between the two countries, and to propose based on the findings an annex or addendum focusing specifically on transboundary aquifers and related dependent ecosystems.

The first step is to analyze the provisions and principles of the 2018 water cooperation and their possible application to the specific characteristics of transboundary aquifers, and highlighting the gaps. These findings will be compared with the conclusions from the Bilateral Groundwater Governance Diagnostic Analysis for the transboundary aquifer Cemi/Cijevna. This comparison will allow to identify more specific definitions, and measures which would need to apply to the case of Cemi/Cijevna. If the principles in the agreement apply generally to transboundary aquifers, the amendment needs to only include specific measures for translating these principles to practical application in the case of Cemi/Cijevna. Such measures do not need to be included in a formal amendment of the bilateral agreement, which will require following specific procedures in each country. Specific and practical measures can be included in annex, requiring certainly simpler formalities for entering into force.

# 3. Current Bilateral Framework (2018 Agreement)

Albania and Montenegro have signed between themselves a framework agreement on “mutual relations in transboundary water management” in 2018. The agreement as its name indicates is a framework, meaning it provides general principles for transboundary water resources, that need afterwards to be declined into more specific measures for implementation.

The agreement refers in its preamble to the two global water conventions: the Convention on the Protection and Use of Transboundary Watercourses and International Lakes (1992) (Water Convention) (both countries are party), and to the Convention on the Law of Non-Navigational Uses of International Watercourses (1997) (only Montenegro is party). Both conventions cover the same core principles of international water law and are complementary. The Water Convention integrates completely transboundary groundwater within its scope, which is not the case of the UN Watercourses Convention. It also refers to the Convention on Environmental Impact Assessment in Transboundary context (Espoo Convention, 1991), and to the Convention for the Protection of the Mediterranean Sea Against Pollution (Barcelona 1976) and all its protocols, which cover not only the sea itself, but also the coastal zones, and all resources in these zones including water.

Under this agreement, Albania and Montenegro commit to the joint management of transboundary water resources, covering activities that impact water quantity and quality. The agreement addresses cooperation in water management, including hydroelectric plants, major lakes and rivers (Shkodra/Skadar, Drin, Buna, Morača), erosion control, and flood protection. Key areas include balancing water resources, preventing harmful effects, maintaining watercourses, pollution protection, shared use of water facilities, and managing waters of shared interest. It also provides for mutual notification, consultation, and cooperation at regional and other organizational levels, with an emphasis on assessing the environmental impacts of water management actions.

The agreement applies to both transboundary surface and groundwaters as per the definition of transboundary waters that it includes, which is the same as the 1992 Water Convention (article 3). Furthermore, the definition of “waters of shared interest” refers also to groundwater. The agreement is based on the equitable and reasonable use and the no harm principles, the two core principles of international water law, and it includes other principles related to environmental protection such as the principles of prevention, precautionary measures without causing damage, and "polluter pays". The agreement establishes the regular exchange of data between the countries, as well as the monitoring to assess the quantity, quality and status of water status. The parties need to agree on the data and information to exchange. The agreement refers also to the WFD for the methodology on the control of water resources quality and adopts its list of hazardous substances. The Parties shall be consulted on the granting of water permits and other water acts. They shall not undertake any unilateral action and shall refrain from taking measures which may cause negative effects or impact on transboundary water resources on the territory of the other Party. Under the agreement, the parties need to encourage the involvement of the concerned stakeholders in the preparation, review and update of any water management plan or project. The agreement creates a Water Resources Management Commission, composed of 10 members from each country. The agreement does not explicit its duties and competences but mentions that they will be defined by the rules of procedure that the Commission will adopt at the first session of its first meeting. The Commission can set up permanent or ad hoc sub-commissions. Its decisions shall be approved unanimously. The Commission must inform and involve the public in its activities in the enforcement of the decisions adopted. The expenses of the meeting are covered by the Party in which territory the meeting is taking place, while the expenses of the representatives are covered by the related countries.

The agreement between Albania and Montenegro represents a solid foundation for bilateral cooperation on water resources. It covers both transboundary surface and groundwater as per the definition of transboundary waters included in its article 3 on definitions (§ 2). The agreement mentions the Cemi/Cijevna rivers among the waters of shared interest between the two countries (article 2), but it does not address the transboundary aquifer. As detailed above, the agreement includes the core principles of international water law which are also applicable to transboundary aquifers following the Draft articles on the law of transboundary aquifers (2018), along with principles related to environmental protection. While it refers to the WFD, the agreement omits the GWD.

The agreement includes provisions for the regular exchange of data and monitoring. Furthermore it establishes an institutional mechanism for cooperation, the Water Resources Management Commission, under which sub-commissions can be set up. It is therefore possible to consider establishing a sub-commission for transboundary aquifers, which can include a working group for  Cemi/Cijevna water system, or a dedicated sub-commission forCemi/Cijevna

Both countries have nominated members for the Commission, 10 members from each country as required by the Agreement. The Commission adopted its rules of procedure at its first meeting held on 19 September 2019 in Podgorica (Montenegro). The rules of procedure entered into force on 22 February 2022 after the two Presidents from each country signed them.

As mentioned in the agreement, the rules define the competencies of the Commission, which are quite wide in matter of cooperation on transboundary water (article 5). The Commission has the general competency of ensuring cooperation and maintaining continuity and unity in water management between the two countries. More specifically, it can:

- Conduct technical and economic studies and research

- Adopt decisions based on these studies

- Develop and evaluate projects

- Propose the implementation and maintenance of common water facilities

- Control their execution, and follow-up on the decisions

- Decide on the exchange of information and mutual consultation

- Propose solutions water resources management issues, and to eventual transboundary impacts

- Monitor the implementation of the decisions it adopted and organize technical control

One important competency of the Convention is the consideration of proposing and harmonizing acts related to its overall competencies, for submission to the competent authorities in Montenegro and Albania.

The Commission holds a specific competence on the Skadar Lake Basin and the Bojana/Buna River with the objective of finding an appropriate management model for the transboundary water management.

And finally, the Commission oversees the work of established sub-commissions, expert groups and experts, which submit reports on their activities. On this basis, the Commission makes the appropriate conclusions on the performed tasks and the planned ones.

The rules of procedure specify the functioning of the Commission, on its sessions, the decision-making process, the costs, and the roles of the Presidents.

The rules of procedure introduce the principle of transparency. The work of the Commission is is public unless the Commission has unanimously decided otherwise.

Under the Water Convention (1992), and with the support of its Implementation Committee, in 2023, experts from Albania and Montenegro discussed needs and ways for establishing adequate monitoring of transboundary groundwaters in the Cemi/Cijevna basin. The experts also discussed with members of the Implementation Committee the need to jointly collect data on groundwater in the basin, develop groundwater flow maps (https://unece.org/media/news/378068). This initiative related to the Cem/Cijevna River involves the development of a Protocol for the exchange of hydrological, water level of groundwater and climatic data. The Protocol is currently pending approval.

In summary, the Framework Agreement between Albania and Montenegro needs to be complemented by a document, which can be in the form of an addendum, or an annex to provide more detailed considerations for transboundary aquifers. It can be an addendum specific to the case of Cemi/Cijevna, referring to the Joint Action Plan, establishing the joint sub-commission for this TBA, or it can be a general addendum on transboundary groundwater, and establishing a sub-commission in charge of groundwater.

# 4. Diagnostic: Groundwater and GDE Governance Gaps

This chapter provides a structured diagnostic of legal, institutional and operational gaps in groundwater and groundwater-dependent ecosystem (GDE) governance within the Montenegro-Albania bilateral framework. The analysis builds upon the findings of the Bilateral Groundwater Governance Diagnostic Analysis (BGGDA) for the Cemi/Cijevna Transboundary Area and examines shortcomings across four key dimensions: legal clarity, operational mechanisms, planning integration, and institutional capacity and compliance.

The diagnostic confirms that both countries have largely harmonized their water legislation with the EU Water Framework Directive (WFD) and the Groundwater Directive (GWD). However, important governance gaps persist at the operational and transboundary levels. These gaps do not primarily stem from the absence of legislation, but from incomplete implementation, limited harmonization of technical procedures, and insufficient institutional coordination in the specific context of transboundary karst aquifers.

## 4.1 Legal Clarity and Scope Gaps

The current bilateral Agreement on Transboundary Water Management (2018) applies to both transboundary surface and groundwater generally and to “waters of common interest” which also covers groundwater. However, the agreement does not explicitly define or operationalize the concept of a transboundary groundwater body.

Key legal clarity gaps include:

- The absence of an explicit definition of shared aquifers or groundwater bodies crossing the state border;

- The absence of a definition of groundwater-dependent ecosystems (GDEs);

- No detailed measures to prevent deterioration of groundwater status at transboundary scale, including the recognition of coordinated sanitary protection zones (SPZ) and extending across borders;

- No joint delineation procedure for cross-border recharge areas.

For example, recharge zones located in one country may directly influence abstraction wells in the other, yet there is no legal requirement for prior notification, joint assessment or harmonized land-use restrictions in such zones.

Furthermore, while both national systems recognize SPZ as a regulatory tool, they apply different methodologies and implementation practices. In Albania, SPZ are legally recognized but often partially implemented, with Zone I most consistently delineated. In Montenegro, SPZ are regulated under water legislation and rulebooks, yet karst-specific methodological guidance remains limited. The absence of cross-border coordination may lead to asymmetrical protection regimes within the same hydrogeological system.

Thus, the first major governance gap concerns insufficient legal specificity and cross-border clarity regarding shared aquifers and their dependent ecosystems.

## 4.2 Monitoring and Data Gaps

The second category of governance gaps relates to operational mechanisms necessary for effective groundwater management.

### 4.2.1 Joint Monitoring Deficit

The BGGDA for Cemi/Cijevna identifies significant asymmetries and limitations in groundwater monitoring:

- Uneven spatial coverage of monitoring points, particularly in upland recharge zones;

- Lack of harmonized monitoring parameters;

- Inconsistent sampling frequency;

- Limited microbiological monitoring in karst systems;

- Insufficient event-based monitoring during high-flow or rainfall periods.

While both countries conduct national groundwater monitoring in line with WFD/GWD requirements, no joint monitoring network exists for shared aquifers. Monitoring stations are designed and operated independently, without systematic coordination or alignment of methodologies.

Karst aquifers, such as those in the Cemi/Cijevna area, are characterized by rapid groundwater flow and high vulnerability to pollution. Contaminants may travel quickly through conduits with limited natural attenuation. Without coordinated monitoring and harmonized parameter lists, transboundary risks remain insufficiently managed.

### 4.2.2 Data Exchange and Interoperability Gaps

Although the 2018 Agreement provides for general data exchange groundwater-specific data sharing remains ad hoc and not structured through:

- Agreed exchange formats;

- Standardized metadata protocols;

- Shared digital platforms;

- Defined notification timelines.

The BGGDA highlights limited interoperability between national databases and insufficient institutionalization of regular technical exchanges. Data standardization, particularly regarding chemical thresholds and quantitative assessment methodologies, remains incomplete.

In the absence of structured exchange mechanisms, risk communication may depend on informal channels rather than institutional procedures, reducing transparency and predictability.

### 4.2.3 Early Warning and Incident Response Gaps

The current bilateral framework does not include groundwater-specific early warning provisions.

In karst systems, contamination events may be:

- Sudden and rainfall-triggered;

- Microbiological in nature;

- Associated with illegal waste disposal or wastewater discharge;

- Difficult to trace after dispersion.

## 4.3 Planning Integration and Spatial Governance Gaps

The third category concerns integration of groundwater protection into spatial and sectoral planning.

The BGGDA indicates that while groundwater is formally recognized within River Basin Management Plans (RBMPs), operational integration into spatial planning instruments remains inconsistent.

Spatial planning procedures are conducted independently at national or municipal levels, without mandatory cross-border consultation and coordination for activities located in hydrogeologically sensitive areas. This absence of coordination may result in:

- Increased pollution risks;

- Uncontrolled development in recharge zones;

- Reduced resilience of groundwater-dependent ecosystems.

## 4.4 Institutional Capacity and Compliance Gaps

No permanent bilateral groundwater sub-commission exists under the Joint Commission framework. As a result, groundwater issues may not receive continuous technical attention.

## 4.5 Synthesis of Governance Gaps

The diagnostic demonstrates that:

- Legislative alignment with EU water acquis is largely achieved;

- However, bilateral groundwater governance remains under-institutionalized;

- Operational joint harmonization is insufficient;

- Karst-specific risk dynamics are not adequately addressed;

- An joint institution (a sub-commission under the Water Commission) is non-existing, with the required technical capacities.

The governance deficit is therefore primarily operational and institutional rather than normative.

Addressing these gaps requires:

- Explicit inclusion of transboundary groundwater bodies and GDEs within the bilateral legal framework;

- Establishment of a joint monitoring system;

- Coordination of SPZ delineation methodologies;

- Structured data exchange protocols;

- Groundwater-specific early warning mechanisms;

- Creation of a permanent technical groundwater sub-group.

Such measures would not represent a fundamental restructuring of the 2018 Agreement but rather a targeted operational enhancement aligned with the findings of the BGGDA and consistent with evolving DIKTAS II governance architecture.

# 5. Evidence from the Cemi/Cijevna TBA: why amendments are needed?

The diagnostic analysis presented in Chapter 4 identifies structural governance gaps in transboundary groundwater management. The Bilateral Groundwater Governance Diagnostic Analysis (BGGDA) for the Cemi/Cijevna Transboundary Area (TBA) provides practical evidence illustrating why these gaps require targeted amendments to the existing bilateral framework.

The Cemi/Cijevna case demonstrates that groundwater governance challenges in Dinaric karst systems are not theoretical. They arise from hydrogeological vulnerability, asymmetric dependency patterns, localized pressure hotspots, and insufficiently operationalized cross-border coordination mechanisms. While the 2018 Montenegro-Albania Agreement establishes a solid foundation for cooperation it does not contain aquifer-specific provisions capable of addressing these challenges in a systematic and preventive manner.

## 5.1 Karst Vulnerability and Transboundary Hydrogeological Connectivity

The Cemi/Cijevna basin is situated within the Dinaric karst region, characterized by highly permeable carbonate formations, complex conduit systems, rapid infiltration, and limited natural attenuation capacity. Hydrogeological processes in such systems differ significantly from those in porous aquifers.

Key characteristics include:

- Rapid transfer of recharge water through fissures and conduits;

- Strong hydraulic connectivity between surface water and groundwater;

- Seasonal sinking of river flows into subsurface systems;

- Recharge zones extending across administrative and state borders.

The BGGDA notes that during low-flow periods, part of the Cijevna River, as well as its karst hinterland, contributes to recharge of the Zeta Plain aquifer, which supplies drinking water to Podgorica, Tuzi and Zeta municipalities. This illustrates a direct hydrogeological link between upstream karst recharge areas and downstream abstraction points.

In such conditions, contamination events-whether microbiological, chemical, or diffuse-may propagate rapidly across borders. Traditional surface-water monitoring regimes are insufficient to capture rainfall-triggered turbidity spikes or episodic microbiological contamination events characteristic of karst systems.

However, the current bilateral framework does not establish:

- A joint karst-adapted monitoring system;

- Event-based sampling obligations;

- Shared thresholds for groundwater contamination;

- A coordinated early-warning mechanism specific to groundwater.

The hydrogeological nature of the Cemi/Cijevna system therefore underscores the need for aquifer-specific amendments.

## 5.2 Drinking Water Dependency and Recharge Area Governance

The Zeta Plain aquifer plays a critical role in municipal water supply in Montenegro. Its recharge dynamics are influenced by karst formations within the broader Cemi/Cijevna basin.

This creates a structural dependency pattern:

- Downstream drinking water security depends on upstream land-use practices;

- Recharge areas may extend beyond immediate administrative control;

- Pollution or over-abstraction in recharge zones can affect abstraction points elsewhere.

Although both countries regulate groundwater abstraction and protection nationally, the BGGDA identifies the absence of a formally recognized joint conceptual hydrogeological model. Without such a model, recharge zones and hydraulic connections are not jointly validated, and cumulative impacts cannot be systematically assessed at aquifer scale.

Furthermore, sanitary protection zones (SPZ) are regulated within national systems but are not harmonized across borders. there is no joint delineation procedure for transboundary recharge zones, nor a mechanism for mutual recognition of protection regimes.

This gap is particularly relevant in karst terrain, where recharge zones may be extensive and diffuse, and where land-use decisions in one jurisdiction may affect water quality and quantity in another.

## 5.3 Pressure Hotspots: Local Activities with Transboundary Effects

The Cemi/Cijevna BGGDA identifies several categories of pressure hotspots that illustrate the limitations of the current governance framework.

### 5.3.1 Peri-Urban and Rural Sanitation Gaps

In upland settlements within the recharge area:

- Wastewater collection infrastructure is incomplete;

- Septic systems are widely used;

- Wastewater treatment capacity remains limited.

In karst systems, untreated or poorly treated wastewater may infiltrate directly into groundwater pathways. While sanitation planning is conducted at municipal or national level, there is no structured mechanism for assessing transboundary groundwater implications of sanitation deficits in recharge zones.

### 5.3.2 Development Nodes

The Cemi/Cijevna canyon and surrounding mountainous landscapes have experienced increased tourism activity. Tourism-related pressures include:

- Seasonal increases in abstraction;

- Increased wastewater production;

- Infrastructure development in hydrogeologically sensitive areas.

Spatial planning and tourism licensing processes are not systematically linked to transboundary groundwater vulnerability assessments. Without harmonized SPZ criteria and shared vulnerability mapping, tourism expansion may increase risks to shared aquifers.

### 5.3.3 Diffuse Agricultural Pressures

Agricultural activities within recharge zones contribute to:

- Nitrate loading;

- Pesticide presence;

- Irrigation abstraction pressures.

Although national legislation addresses agricultural pollution control, there is limited coordination regarding vulnerability mapping and land-use compatibility criteria for transboundary recharge areas. Monitoring data are not systematically harmonized to detect long-term trends across the border.

### 5.3.4 Identification of Illegal or Unregistered Abstractions

The BGGDA also notes challenges in identifying and registering existing groundwater abstractions within river basins. In a shared aquifer context, uncoordinated abstraction may affect quantitative status cumulatively, yet there is no bilateral mechanism for consolidated abstraction assessment.

## 5.4 Governance Frictions and Agreement Limitations

The Cemi/Cijevna TBA reveals specific governance frictions not resolved under the current bilateral Agreement:

- No explicit recognition of shared groundwater bodies;

- No harmonized monitoring parameter list;

- No joint monitoring network;

- No aquifer-level reporting obligation;

- No groundwater-specific early-warning protocol;

- No permanent bilateral groundwater technical subgroup;

- No coordination on cross-border SPZ mechanisms.

These frictions do not stem from lack of national legislation. Rather, they result from insufficient operationalization of groundwater governance at transboundary level.

The Cemi/Cijevna example demonstrates that:

- Karst aquifers require tailored governance instruments;

- Drinking water security depends on coordinated recharge area protection;

- Localized pressure hotspots may have transboundary consequences;

- Surface-water oriented cooperation frameworks do not automatically ensure groundwater protection.

## 5.5 Conclusion: Evidence-Based Justification for Amendments

The evidence from the Cemi/Cijevna TBA confirms the conclusions of Chapter 4: groundwater governance deficits are primarily operational and institutional.

Amendments to the 2018 Agreement are therefore justified in order to:

- Provide an explicit definition of transboundary groundwater bodies and GDEs;

- Establish a joint monitoring system adapted to karst dynamics;

- Harmonize SPZ delineation methodologies;

- Introduce groundwater-specific early-warning procedures;

- Create a permanent bilateral groundwater technical mechanism.

The Cemi/Cijevna case provides a practical, evidence-based demonstration that strengthening groundwater governance is necessary to ensure long-term drinking water security, ecosystem stability, and compliance with EU water acquis in transboundary karst contexts.

# 6. Objectives for an Agreement Addendum/Protocol

Considering the governance gaps identified in Chapter 4 and the practical evidence derived from the Cemi/Cijevna TBA (Chapter 5), the proposed Addendum or Protocol to the 2018 Montenegro-Albania Agreement should pursue clearly defined, measurable and operational objectives. These objectives are designed to transform general cooperation principles into structured aquifer-level governance mechanisms.

The objectives below are formulated in amendment-ready language.

Objective 1: Explicit Recognition of Shared Groundwater Bodies and Groundwater-Dependent Ecosystems

The Parties shall explicitly recognize transboundary groundwater bodies and groundwater-dependent ecosystems (GDEs) hydraulically connected across their territories.

To this end, the Addendum shall:

- Define “Transboundary Groundwater Body” and “Groundwater-Dependent Ecosystem” in line with EU WFD/GWD terminology;

- Acknowledge the hydrogeological interconnectivity of karst aquifers within the Cemi/Cijevna basin;

- Establish the obligation to prevent deterioration of quantitative and chemical status of shared groundwater bodies;

- Apply the principles of equitable and reasonable use and no significant harm specifically to groundwater resources.

Measurable outcome: The Parties shall adopt a jointly validated delineation of shared groundwater bodies and associated recharge zones.

Objective 2: Establishment of a Joint Groundwater Monitoring and Data Exchange System

The Parties shall establish a harmonized joint groundwater monitoring system adapted to karst hydrogeological conditions.

The Addendum shall provide that:

- A joint monitoring network shall be designated, including agreed monitoring points on both sides of the border;

- A harmonized parameter list shall be adopted, covering quantitative, chemical and microbiological indicators;

- Sampling frequency and event-based monitoring (particularly rainfall-triggered events) shall be standardized;

- Quality assurance and quality control (QA/QC) procedures shall be harmonized;

- Data shall be exchanged quarterly in a mutually agreed format;

- Immediate notification shall occur in case of agreed threshold exceedance.

Measurable outcomes: Adoption of a Joint Monitoring Protocol, Operational data exchange platform, Annual joint groundwater status report prepared and submitted to the Joint Commission.

Objective 3: Early-Warning and Incident Response Mechanism for Groundwater

The Parties shall establish a groundwater-specific early-warning and incident response protocol.

The Addendum shall provide that:

- Threshold values triggering notification shall be jointly defined;

- Notification shall occur within 24 hours of detection of significant contamination or abnormal turbidity/microbiological event;

- A joint technical assessment shall be conducted where transboundary impact is likely;

- Temporary risk mitigation measures (including abstraction adjustments, where necessary) shall be coordinated;

- A post-incident report shall be prepared and reviewed by the Joint Commission.

Measurable outcomes: Adoption of Early-Warning Protocol; Designation of official groundwater contact points in each Party, Annual summary of incidents.

Objective 4: Institutionalization of Periodic Joint Reporting and Review

The Parties shall institutionalize regular review of groundwater governance under the Joint Commission framework.

By translating general cooperation principles into structured aquifer-specific obligations, the proposed Addendum would strengthen legal certainty, improve risk prevention, and enhance long-term drinking water security and ecosystem resilience in transboundary karst contexts.

# 7. Proposed Legal Amendments and Key Action Points

7.1 Include definitions such as “Transboundary Groundwater Body”, “Groundwater‑Dependent Ecosystem”, “Sanitary Protection Zone (SPZ)”, “Incident”.

7.2 Principles & Scope

The framework agreement between Albania and Montenegro already includes the core principles of international water law for transboundary cooperation, the equitable and reasonable utilization and the no harm rule, as well as principles related to precaution, and protection. It also provides for the regular exchange of data and monitoring.

However, the principles on precaution and protection remain broad, and merits to be more specific for the case of the Cemi/Cijevna transboundary aquifer, and indicate the protection of ecosystems dependent upon the Cemi/Cijevna, with measures related to ensuring that the quality and the quantity of the water remains sufficient to protect and preserve these ecosystems. Such measures can include the identification of Sanitary Protection Zones. This need to be specified in the addendum.

7.3 Joint Monitoring System (network design, priority springs/wells; harmonized parameters, frequency; microbiology/turbidity for karst; QA/QC; shared digital platform and INSPIRE‑compatible metadata).

7.4 Data Exchange & Confidentiality (near‑real‑time exchange; validation; annual public summaries).

7.5 SPZ updating and integration (four‑zone model for karst; vulnerability mapping requirement; land‑use compatibility lists; integration into spatial plans on both sides within specified deadlines).

7.6 Early‑Warning & Incident Management (thresholds, contacts, timelines, temporary abstraction curtailment, public notice, post‑event joint report).

7.7 Institutional Arrangements (groundwater sub‑commission under the Joint Commission; annual technical reviews; stakeholder engagement).

The 2018 Framework agreement establishes a Water Resources Management Commission, under which sub-commissions can be created. The Commission has already been established with members nominated in both countries. However, its duties and competences are not defined yet. Nevertheless, it can be envisaged to create a sub-commission specifically for transboundary groundwater including a sub-group for the Cemi/Cijevna basin, covering surface waters and groundwaters as it seems difficult to separate both. Moreover, the sub-commission could oversee the implementation of the Joint Action Plan.

7.8 Financing & Economic Instruments (principles) (earmarking for monitoring/SPZ implementation; cooperation on remedial measures).

7.9 Implementation, Reporting & Review (biennial implementation report; five‑year clause review).

7.10 Relationship to Existing Instruments (consistency with 1996 Agreement; without prejudice clauses)

The requirements for the addendum fall well under the Framework agreement between Albania and Montenegro on transboundary water resources. It complements and specifies the cooperation and protection principles of the agreement by providing application measures adapted to transboundary groundwater bodies, and moreover in a karstic environment.

# 8. Implementation Roadmap (Phased, 36 months)

Three phases with deliverables and responsible bodies:

- Phase 1 (0-12 months): adopt Addendum/Protocol; draft SPZ mutual‑recognition guidance; set up emergency notification template.

- Phase 2 (12-24 months): deploy shared data platform; align permit templates; integrate SPZ/vulnerability layers into local plans in designated hotspots.

- Phase 3 (24-36 months): adopt cost‑recovery earmarking; publish first joint implementation report.

# 9. Risks and Mitigation Measures

The implementation of the proposed amendments may involve several operational and institutional risks, which can be addressed through appropriate mitigation measures.

| Identified Risk | Description | Proposed Mitigation Measures |
| --- | --- | --- |
| Institutional fragmentation | Responsibilities for groundwater governance are distributed among several institutions at national and local levels, which may lead to coordination gaps and unclear accountability. | Establish clear institutional mandates, define reporting lines under the Joint Commission, and create a permanent bilateral technical sub-group for groundwater cooperation. |
| Data incompatibility | Monitoring systems, methodologies and databases may differ between the two countries, limiting comparability and joint assessment of groundwater status. | Adopt a harmonized monitoring parameter list, standardized sampling methodologies, and agreed QA/QC procedures, supported by a shared data exchange platform. |
| Financial constraints | Implementation of joint monitoring networks, hydrogeological studies and data management systems requires sustained financial resources. | Apply a phased implementation approach and explore external funding opportunities through EU, IPA, or regional cooperation programmes. |
| Political sensitivity | Transboundary water management may occasionally involve political sensitivities, particularly where resource dependency is uneven. | Maintain the process within a technical and expert-driven framework, ensuring transparent communication and regular consultations under the Joint Commission. |
| Karst hydrogeological uncertainty | Karst aquifers exhibit complex and sometimes unpredictable flow dynamics, making hydrogeological interpretation and risk prediction challenging. | Apply adaptive management principles, periodically review monitoring networks and conceptual models, and incorporate new scientific data into decision-making processes. |

# 10. Conclusions and Next Steps

The amendment of the 2018 Montenegro-Albania Agreement to explicitly regulate transboundary groundwater is legally justified, technically necessary, and institutionally feasible. It basically needs to explicit the principles from the agreement to practical applicability to transboundary groundwater bodies in general, which afterwards can be detailed for the specific case of Cemi/Cijevna transboundary water system, probably in the Joint Action Plan. The amendments can be adopted in the form of an annex and do not necessitate the revision of the Agreement.